Analysis of impurities in pure and balance gases used to prepare primary standard gas mixtures by the gravimetric method
R.J. Oudwater1, J.I.T. van Wijk1, S. Persijn1, R.M. Wessel1, A.M.H. van der Veen1*, T. Mace2, C. Sutour2, J. Couette2, M. Milton3, A. Harling3, G. Vargha3, I. Uprichard3, H.-P. Haerri4 and B. Niederhauser4, D. Tuma5, M. Maiwald5, C. Boissière5
1VSL, Thijsseweg 11, 2629 JA Delft, the Netherlands, avdveen@vsl.nl
2Laboratoire National de Métrologie et d’Essais (LNE), 1, rue Gaston Boissier, 75724 Paris Cedex 15
3National Physical Laboratory (NPL), Teddington, Middlesex, TW11 0LW, UK
4Federal Institute of Metrology METAS, Lindenweg 50, 3003 Bern-Wabern, Switzerland
5Federal Institute of Materials Research and Testing (BAM), Unter den Eichen 87, 12205 Berlin, Germany
Abstract

This supplementary comparison (Euramet.QM-S8) concerns the purity analysis of nitrogen as used in reference gas mixture preparation. This project was carried out without adding impurities to the gas used for this comparison, and is therefore more representative to evaluate the analysis of carbon monoxide, carbon dioxide, methane, oxygen, argon, and water impurities in high-purity nitrogen. The analysis of the amount–of–substance fraction water was optional.
Two 50 litre high purity nitrogen cylinders were purchased from a well-qualified supplier of specialty gases. The listed components were expected to be present in the pure nitrogen at the target levels as a result of the purification of the nitrogen. From the start of this comparison it was clear that the comparison may not lead to reference values for the constituents analysed.
The results indicate that analyses of high purity gases are often limited by the limits of detection of analytical equipment used. The reports of the participating laboratories also indicate that there is no agreed method of determination of the uncertainty on a detection limit value. The results provide useful information on the performance of participants. For all analysed components there is reasonable agreement in results for LNE, VSL, METAS and NPL. For BAM only the Argon result is in agreement.
Main text. To reach the main text of this paper, click on Final Report. Note that this text is that which appears in Appendix B of the BIPM key comparison database kcdb.bipm.org/.

The final report has been peer-reviewed and approved for publication by the CCQM, according to the provisions of the CIPM Mutual Recognition Arrangement (MRA).

PAGE  
1
Version 00/00/0000 00:00

